Download the Arduino IDE from https://www.arduino.cc/en/Main/Software

And choose you’r operating system from the list

Go to https://wiki.wemos.cc/tutorials:get_started:get_started_in_arduino

And copy the link http://arduino.esp8266.com/versions/2.3.0/package_esp8266com_index.jsonin the additiona board manager url (witch can be found under files -> preferences )

The requird libary’s fount in the arduno libary maneger

Adafruit  IO arduino

Adafruit MQTT

Arduino http client

How to create tan adafruit account:

Go to : https://accounts.adafruit.com/users/sign_up and singe up for a free account

Then go to io.adafruit.com and singe in with the account created in the previous step

After you singed in to adafruit IO  go to the dasboards tab and creat a new one and name it temperature for example

After you created you’r new dashboard click on the name and add a graph and a stream by clicking on add  new block

After you created you’r dashboard you need to generate a key for you’r code to acsess the service  by clicking on the key symbol and take note of it  and also take note of you’r user name

After that go to https://github.com/bendelathouwer/arduino_Wifi_Datastream

And  press the green download button and download the zip file and unzip the file

Open the Arduino file

The you need to go to the config.h file and fill in you’r Adafruit IO username and  key also you need to fill in you’r WIFI’s ssid and password

Go to the board selector and choose you’r WEEMOS D1 R2 board and hit compile and upload

Enjoy you’r result